L ouisiana State inquiries via Datamaxx Message Broker.

| Date | Versión | Description | By |
| --- | --- | --- | --- |
| 12/13/2021 | 1.0 | Manual | Support team |

Index

1	Datamaxx client installation	4

1.1	Copy the Datamaxx client installer located in the storage path:	4

1.2	Paste the installer into any folder on BizTalk server (10.255.255.57) and run the installation.	4

1.3	Once the installation has finished, the destination folder contains:	7

2	Datamax client configuration.	8

2.1	DMX_WS_Client_Config installer.	8

2.2	DMX_WS_Client_Config.exe	9

2.3	Service.	10

3	State interface:	12

Configuration on server 192.255.255.80	12

3.1	State interface configuration file.	12

3.2	State interface service.	13

4	Support contacts	15

4.1	St. Tammany Sheriff's Office IT:	15

4.2	Datamaxx Support	15

OBJECTIVE

This document presents the steps for the installation and configuration of Datamaxx Broker for integration with the state query service from St Tammany and Covington.

# Datamaxx client installation

## Copy the Datamaxx client installer located in the storage path:

\\storage\Public Safety\Customer Services Division\Customer Support\Customer Support Knowledge Base\Support Related Documentation (INTERNAL)\#Customer Specific Environment Information\STPSO\Datamaxx LA State interface\Installer

## Paste the installer into any folder on BizTalk server (10.255.255.57) and run the installation.

You will be prompted with the following message, so click on run Anyway

Accept the EULA and click on Next button.

Do not change the default destination folder and click on Next button.

Click on Install button.

## Once the installation has finished.

Path: C:\Program Files (x86)\Datamaxx Applications\DMX_WS_Client

Destination folder should contain:

# Datamax client configuration.

## DMX_WS_Client_Config installer.

Run the installer as administrator and input the values exactly as shown. (Values provided by LA State an Datamaxx support)

Client IP port: You can use any port number; you must make sure that the one chosen will be the same in all configurations.

If there are prior saved values, click on cancel button to load them.

## Datamaxx service configuration.

From Command Prompt (cmd) go to the Datamaxx default installation folder and run the DMX_WS_Client_Config.exe file. with the (/R) option to generate a response file. See below.

## Service.

After completing the installation, open services manager and make sure let this service configured as shown.

After 5 minutes the service should be up, service’s logs must appear like this.

Datamaxx server should be connected with the CAD server and the state interface status UP.

# State interface:

## Configuration on server 192.255.255.80

## State interface configuration file.

Open the StateInterfaceWinService.exe.Config file from the Path: C:\Program Files (x86)\MobileTec International\StateInterface and be sure it has these parameters.

.

Local and remote port has to be the same choosed in step 2.1 DMX_WS_Client_Config installer.

## State interface service.

This must be configurated in this way.

Restart this service and after 5 minutes the service should be up.

To confirm the service status and it is working properly, please go to the following path and check Trace.log file must appear like next images.

Path: C:\mobiletec\server\logsws\StateInterfaces

A HeartBeat is sent to the State Interface which replies to this server; both message logs must not show any error.

# Support contacts

## St. Tammany Sheriff's Office IT:

Pete Vicknair,  PeteVicknair@stpso.com

James E. Jones,  jamesjones@stpso.com

## Datamaxx Support

Ardney Boland

Technical Support Specialist III

Ardney.Boland@datamaxx.com

Direct: 850-558-8108

Technical Support Phone: 877.369.8324

Technical Support Email: support@datamaxx.com